#include <Wire.h>                // Library for I2C communication

#include <LiquidCrystal_I2C.h>   // Library for I2C LCD

#include <Keypad.h>              // Library for keypad

#include <Servo.h>               // Library for servo motor

/* LCD */

LiquidCrystal_I2C lcd(0x27, 16, 2);  // LCD address 0x27, 16 columns, 2 rows

/* Servo */

Servo lockServo;                 // Create servo object

#define SERVO_PIN 9              // Servo connected to pin 9

#define LOCK_POS 10              // Servo angle for locked position

#define UNLOCK_POS 90            // Servo angle for unlocked position

/* Keypad */

const byte ROWS = 4, COLS = 4;   // 4 rows and 4 columns keypad

// Keypad button layout

char keys[ROWS][COLS] = {

{'1','2','3','A'},

{'4','5','6','B'},

{'7','8','9','C'},

{'*','0','#','D'}

};

// Arduino pins connected to keypad rows

byte rowPins[ROWS] = {2,3,4,5};

// Arduino pins connected to keypad columns

byte colPins[COLS] = {6,7,8,10};

// Create keypad object

Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, ROWS, COLS);

/* PIN */

String PIN = "2580";   // Default PIN

String input = "";          // Stores entered numbers

bool unlocked = false;      // Check if door is unlocked

bool changeMode = false;    // Check if PIN change mode is active

void setup() {

lcd.init();               // Start LCD

lcd.backlight();          // Turn ON LCD light

lockServo.attach(SERVO_PIN);  // Attach servo to pin 9

lockServo.write(LOCK_POS);    // Start with locked position

lcd.print("Enter PIN");  // Show message on LCD

}

void loop() {

char key = keypad.getKey();   // Read pressed key

if (!key) return;             // If no key pressed, do nothing

/* LOCK when D pressed */

if (key == 'D' && unlocked) {     // If D pressed and door is unlocked

lockServo.write(LOCK_POS);      // Move servo to lock position

unlocked = false;              // Set unlocked status to false

lcd.clear();                   // Clear LCD

lcd.print("Locked");           // Show locked message

delay(1000);                   // Wait 1 second

lcd.clear();

lcd.print("Enter PIN");   // Ask for PIN again

return;                        // Exit loop

}

/* Backspace */

if (key == 'C' && input.length() > 0) {   // If C pressed

input.remove(input.length()-1);         // Remove last digit

lcd.setCursor(0,1);

lcd.print("                ");          // Clear second row

lcd.setCursor(0,1);

for(int i=0;i<input.length();i++)

lcd.print("*");                       // Show stars again

return;

}

/* Change PIN */

if (key == 'A' && !unlocked) {     // If A pressed and door locked

lcd.clear();

lcd.print("Old PIN:");           // Ask for old PIN

input = "";                      // Clear input

changeMode = true;               // Activate change mode

return;

}

/* Number Input */

if (isdigit(key) && input.length() < 4) {  // If number pressed

input += key;                            // Add number to input

lcd.setCursor(input.length()-1,1);

lcd.print("*");                           // Show * instead of number

}

/* When 4 digits entered */

if (input.length() == 4) {

delay(300);   // Small delay

if (changeMode) {     // If PIN change mode active

if (input == PIN) {   // Check old PIN correct

lcd.clear();

lcd.print("New PIN:");   // Ask for new PIN

input = "";

while (input.length() < 4) {   // Take 4 digit new PIN

char k = keypad.getKey();

if (isdigit(k)) {

input += k;

lcd.setCursor(input.length()-1,1);

lcd.print("*");

}

}

PIN = input;         // Save new PIN

lcd.clear();

lcd.print("PIN Changed");

delay(1500);

}

else {   // If old PIN wrong

lcd.clear();

lcd.print("Wrong PIN");

delay(1500);

}

changeMode = false;   // Exit change mode

}

else {   // Normal unlock mode

if (input == PIN) {   // If PIN correct

lockServo.write(UNLOCK_POS);  // Unlock door

unlocked = true;             // Set unlocked status

lcd.clear();

lcd.print("Unlocked");

lcd.setCursor(0,1);

lcd.print("Press D=Lock");   // Tell user how to lock

input = "";

return;

}

else {   // If PIN wrong

lcd.clear();

lcd.print("Wrong PIN");

delay(1500);

}

}

input = "";                 // Clear input

lcd.clear();

lcd.print("Enter PIN");   // Ask again

}

}